Lab 7:	Content Distribution

Name:

Date:

Objectives

Objective:  Learn how content distribution works

- Review lab6 for DNS and HTTP servers

- Learn to use the command wget to support content distribution

# Background Reading

Read sections about content distribution.  Review lab6 for how to set up a DNS server and a HTTP server.  Read the man pages on wget.

# Required Equipment

- Operational networks

- Three DNS servers

- Three web servers

# Excercises

Students will perform the following tasks and explain:

a)How they did them.

b)Why they were/were not successful.

c)What they observed.

1____Content Distribution

In order to simulate a CDN we performed the following procedures.

- setup an ip alias to identify requests from different networks

ifconfig eth0:1 192.168.10.4 netmask 255.255.255.0

- configured an additional DNS server for the other network

tinydns-conf   tinydns  dnslog  /var/djbdns/tinydns2

ln  -s  /var/djbdns/tinydns2  /service

cp /var/djbdns/tinydns/root/data /var/djbdns/tinydns2/root/data

cd  /var/djbdns/tinydns2/root

add-host  cdn.omega.com  192.168.1.4

make

cd /var/djbdns/tinydns/root

add-host cdn.omega.com   192.168.2.4

make

- configured route for prerouting

route add –host  192.168.10.4 netmask 255.255.255.0  eth0:1

- setup prerouting to discriminate requests

iptables  -A  PREROUTING  -s 192.168.1.0/24 –d 192.168.10.5 –p UDP

--dport 53  -j DNAT  -t nat  --to-destination  192.168.10.4

use the following diagram for this lab

- start the web server on each of the two networks(see lab 6)

- create a simple web page on the Content Distribution Network host 192.168.10.6  Name the file index.html

- add a link in the index page to an image file from the content distribution site cdn.omega.com

- on each sub-network’s web server perform wget on the CDN’s web page

root# wget   -k --mirror  -- span-hosts  www.omega.com

- verify the retrieval of the images from the respective network

--12:02:03--  http://www.omega.com/

=> `www.omega.com/index.html'

Resolving www.omega.com... done.

Connecting to www.omega.com[192.168.10.6]:80... connected.

HTTP request sent, awaiting response... 200 OK

Length: 147 [text/html]

The sizes do not match (local 143) -- retrieving.

--12:02:03--  http://www.omega.com/

=> `www.omega.com/index.html'

Connecting to www.omega.com[192.168.10.6]:80... connected.

HTTP request sent, awaiting response... 200 OK

Length: 147 [text/html]

100%[========================================>] 147          143.55K/s    ETA 00:00

12:02:03 (143.55 KB/s) - `www.omega.com/index.html' saved [147/147]

Loading robots.txt; please ignore errors.

--12:02:03--  http://cdn.omega.com/robots.txt

=> `cdn.omega.com/robots.txt'

Resolving cdn.omega.com... done.

Connecting to cdn.omega.com[192.168.1.4]:80... connected.

HTTP request sent, awaiting response... 404 Not Found

12:02:03 ERROR 404: Not Found.

--12:02:03--  http://cdn.omega.com/some.png

=> `cdn.omega.com/some.png'

Connecting to cdn.omega.com[192.168.1.4]:80... connected.

HTTP request sent, awaiting response... 200 OK

Length: 3,613 [image/png]

Server file no newer than local file `cdn.omega.com/some.png' -- not retrieving.

FINISHED --12:02:03--

Downloaded: 147 bytes in 1 files

Converting www.omega.com/index.html... 1-0

Converted 1 files in 0.00 seconds.

- run traceroute to the following locations to illustrate the usefulness

What are the time differences if transmissions?

local network’s web server

CDN network

Other network’s web server