Lab 8:	Clustering

Name:

Date:

Objectives

Objective:  Set up a virtual server to support clustering

- Build a Linux kernel

- Configure the kernel for virtual server support

- Uses IPVSADM tools to set up a cluster

# Background Reading

Read sections about clustering and review the Linux LVS documentation.

# Required Equipment

- Seven PC’s

- A switch with a least seven ports

- A router with two available Ethernet interfaces

# Excercises

Students will perform the following tasks and explain:

a)How they did them.

b)Why they were/were not successful.

c)What they observed.

Use the following diagram for the lab.

1____Design and setup the network for clustering

- Follow the diagram to set up the network.

- Reconfigure the router interfaces.

- Configure the client host machine interface.

2____Retrieve the kernel and necessary patches

- copy and untar the tarred kernel file to /usr/src

- make a symbolic link to the new kernel directory

- copy the kernel patches to /usr/src/linux

- apply the patches

patch  -p1  <  linux-2.4.18-ipvs-1.0.2.patch

(do not apply the hidden patch on the director)

patch  -p1  <  hidden-2.4.5-1.diff

- Configure the kernel for ipvs by selecting the following menu items

root# make menu config

1.	file system support

EXT3

2.	network support

Ethernet 10/100

3COM

3C590 or 3C900

3.	network options

add everything

4.	network filter

connection tracking

add everything

5.	virtual server configuration

add everything

- Build the kernel using the following commands

root# make dep

root# make clean

root# make bzImage

root# make modules

root# make install

- Modify the bootloader to use the new kernel on reboot

root#  cp  linux/arch/i386/boot/bzImage  /boot

root#  cd /boot

root#  rm vmlinuz

root#  mv  bzImage  bzImage-2.4.19

root#  ln  -s  bzImage-2.4.19  vmlinuz

- Edit the menulist for grub with the following changes

root#  vi  /boot/grub/menulist

kernel  vmlinuz   root  =  /dev/hda2

(remove the initrd line)

- Reboot the machine

root#  reboot

3____Install and configure ipvsadm tools

- Copy the ipvsadm tools to /usr/src

- Change directories to ipvsadm

root#  cd  ipvsadm

- Install ipvsadm

root#  make install

- Copy the real server script to /root on all real servers

root#  cp  lvs-rsv3.conf  /root

- Copy the virtual server script to /root on the director

root#  cp  lvs-d.conf  /root

- Edit the server scripts accordingly

LVS-D.CONF

#!/bin/bash

#---------------mini-rc.lvs_dr-director------------------------

#set ip_forward OFF for vs-dr director (1 on, 0 off)

/sbin/ifconfig eth0 192.168.3.2 broadcast 192.168.3.255 netmask 255.255.255.0

/sbin/route add -host 192.168.3.2 dev eth0

echo "0" > /proc/sys/net/ipv4/ip_forward

#director is not gw for realservers: leave icmp redirects on

echo 'setting icmp redirects (1 on, 0 off) '

echo "1" > /proc/sys/net/ipv4/conf/all/send_redirects

cat       /proc/sys/net/ipv4/conf/all/send_redirects

echo "1" > /proc/sys/net/ipv4/conf/default/send_redirects

cat       /proc/sys/net/ipv4/conf/default/send_redirects

echo "1" > /proc/sys/net/ipv4/conf/eth0/send_redirects

cat       /proc/sys/net/ipv4/conf/eth0/send_redirects

/sbin/ifconfig eth0:1 192.168.3.1 broadcast 192.168.3.1 netmask 255.255.255.255

/sbin/route add -host 192.168.3.1 dev eth0:1

/sbin/route add default gw 192.168.3.254 dev eth0

#setup_ipvsadm_table

#clear ipvsadm table

/sbin/ipvsadm -C

#installing LVS services with ipvsadm

#add telnet to VIP with round robin scheduling

/sbin/ipvsadm -A -t 192.168.3.1:http -s rr

#forward http to realserver using direct routing with weight 1

/sbin/ipvsadm -a -t 192.168.3.1:http -r 192.168.3.3 -g -w 1

/sbin/ipvsadm -a -t 192.168.3.1:http -r 192.168.3.4 -g -w 1

/sbin/ipvsadm -a -t 192.168.3.1:http -r 192.168.3.5 -g -w 1

/sbin/ipvsadm -a -t 192.168.3.1:http -r 192.168.3.6 -g -w 1

#displaying ipvsadm settings

/sbin/ipvsadm

#not installing a default gw for LVS_TYPE vs-dr

#---------------mini-rc.lvs_dr-director------------------------

LVS-RSV3.CONF

#!/bin/bash

#----------mini-rc.lvs_dr-realserver------------------

#installing default gw 192.168.3.254 for vs-dr

/sbin/ifconfig lo 127.0.0.1 broadcast 127.255.255.255 netmask 255.0.0.0

/sbin/ifconfig eth0 192.168.3.6 broadcast 192.168.3.255 netmask 255.255.255.0

/sbin/route add default gw 192.168.3.254

#checking if DEFAULT_GW 192.168.3.254 is reachable

ping -c 1 192.168.3.254

#set_realserver_ip_forwarding to OFF (1 on, 0 off).

echo "0" > /proc/sys/net/ipv4/ip_forward

cat       /proc/sys/net/ipv4/ip_forward

#looking for DIP 192.168.1.9

ping -c 1 192.168.3.2

#looking for VIP (will be on director)

ping -c 1 192.168.3.1

#install_realserver_vip

/sbin/ifconfig lo:1 192.168.3.1 broadcast 192.168.3.1 netmask 255.255.255.255

#ifconfig output

/sbin/ifconfig lo:1

#installing route for VIP 192.168.3.1 on device lo:1

/sbin/route add -host 127.0.0.1 dev lo

/sbin/route add -host 192.168.3.1 dev lo:1

#hiding interface lo:1, will not arp

echo "1" > /proc/sys/net/ipv4/conf/all/hidden

cat       /proc/sys/net/ipv4/conf/all/hidden

echo "1" > /proc/sys/net/ipv4/conf/lo/hidden

cat       /proc/sys/net/ipv4/conf/lo/hidden

#----------mini-rc.lvs_dr-realserver------------------

- Stop the network

root#  /etc/init.d/network  stop

- Start the LVS scripts

root#  /root/lvs-d.conf  (on director)

root#  /root/lvs-rsv3.conf  (on real servers)

- Create a text file on each host to reflect host information

root#  ifconfig eth0  >  /var/www/html/text.txt

- Test cluster by repeatedly accessing the cluster site and viewing the text.txt file.