# Testing Efficiency of FTP between a Switch and a Router

Name:

Date:

# Objectives

Objective:  Compare time required to transer a file thru a router and thru a switch.

# Background Reading

Review previous labs, and how to FTP, establish a lan, and set up minicom.

# Required Equipment

- Two established LANs

- One Router

- One Switch

- Cables

# Procedure

- Setup LAN connections

  - Set up the physical connections for the LAN’s.

  - Connect the two computers to the switch. Configure the IP addresses for each computer using the command:  ifconfig 192.168.2(1).3 netmask 255.255.255.0.

  - Test for connectivity by pinging a host.

The following diagram illustrates the setup:

192.168.2.3

switch

192.168.1.3

  - Make sure FTP is running on one of the computers.  A user must be created because FTP will not allow root to transfer files.  Setup a user name on both computers.

  - A file was installed in /home/mmathieu called linux-2.4.19.tar.bz2 that was 26042494 bytes.  A large file is needed to allow for variation of times to be measured over the network.

  - Transfer the file using ftp.  The following commands were used:

ftp 192.168.2.3

login: mmathieu

password: xxxxx

cd /home/mmathieu

ls (Verify file is there)

get linux-2.4.19.tar.bz2

The file was transferred 5 times with the following recorded transfer times:

2.23 s       11.4e+03Kbytes/sec

2.23 s       11.4e+03Kbytes/sec

2.22 s       11.4e+03Kbytes/sec

2.24 s       11.4e+03Kbytes/sec

2.63 s        9.67e+03Kbytes/sec

  - Next, setup the following network:

192.168.2.3

router

192.168.1.3

192.168.2.254

192.168.1.254

  - Review previous labs to configure the interfaces of the router.  Also, minicom may have to be installed to communicate with the router.

  - Following the same format as above, the transfer time between the LANs using FTP were:

3.5 s      7.27e+03Kbytes/sec

5.3 s      4.80e+03Kbytes/sec

6.5 s      3.91e+03Kbytes/sec

6.8 s      3.74e+03Kbytes/sec

5.9 s       4.31e+03Kbytes/sec

### Conclusion

The FTP transfer time is shorter thru a switch  than thru a router.  This conclusion is reached by the shorter transfer time and higher throughput of the switch versus the router.  This is possibly due to less overhead required by a switch at the data link layer than at the network layer for the router.